"dennis kleitsch" <dkleitsch@yahoo.com

Subject: Re: McIntosh caps

To: "Doug Olitsky" <doug_olitsky@yahoo.com>

djk: This amp is old enough to justify replacing the electrolytics in the signal path with new Panasonic non-polar types (Digi-Key).

Input board:

C1, C2, 10µF, add 0.033~0.1µF film (tight fit).

Chassis:

Add 47µF~100µF 100V to C7, C8, 39,000µF (will affect the bass 'slam' quite a bit).

Should I use Panasonic non-polar here too?  add in parallel to each not series?

Driver Board:

C202 (I/P), 2.2µF, add 0.1µF,

C2, 47µF (F/B), add 0.1µF,   is there a C2  here?

C211, 47µF (I/P), add 0.1µF,

C216,47µF (I/P), add 0.1µF,

C216, 100µF (F/B), add 0.1µF, two C216?

C225, 47µF (P/S), add 0.1µF,

C223 (bias), change to 2µF film (or 10µF electro plus 0.1µF, check for fit).

Output board:

C229 (225), 0.047µF, change to the highest quality and voltage that will fit,

C230 (226),0.1µF, change to the highest quality and voltage that will fit.

This is the bang-for-the-buck mod. Beyond this the gains are smaller and cost more.

Invert amp on a piece of carpet before removing the cover. Do not pick up amp with cover off or the glass will BREAK.

http://www.diyaudio.com/forums/showthread/t-64781.html